CVEN 5534 Wastewater Treatment

Assignment 2

Due: Tuesday, Sept. 27

- A municipal wastewater treatment plant receives 70 m3/min of medium strength wastewater (see characterization table in class notes).  Approximately 95% of the influent ammonia is removed before discharge. Of the fraction removed, 80% of the influent NH4-N is oxidized by bacteria to NO3 and 20% is incorporated into bacterial bi

- omass.

- What is the mass flow rate of oxygen (kg/day) that must be supplied for ammonia removal?

- What is the biomass generation rate (kg/year) cell solids if the chemical for bacterial cells is C5H7NO2?

- Show by means of comparing the oxidation of a simple carbohydrate, CH2O, that the oxygen equivalence of nitrate when nitrate is the electron acceptor in respiration = 2.86 g-O2/g-NO3-N.

- The maximum growth yield for a culture of bacteria growing on soluble COD = 0.4 g-cells (as COD)/g-substrate COD. However, it is assumed that 25% of the cells grown eventually die and lyse during the experiment and 50% of the lysis products are available substrate for new cell growth. The remaining 50% of the lysis products are nonbiodegradable organic debris.

  - What is the observed cell yield (COD basis)?

  - What fraction of the total COD at the end of the experiment are debris, assuming all the substrate COD is consumed?

- A municipal treatment plant receives an average wastewater flow of 50,000 m3/d. The influent COD is 300 g/m3, and the observed growth yield for the population of bacteria in the plant is 0.45 g-cells/g-COD.

  - Calculate the observed yield on a COD basis.

  - Write the COD-based stoichiometry for oxidation of the influent organic matter (COD).

  - Calculate the mass of oxygen required per day to stabilize the waste.

  - What is the rate at which viable cells are grown (in terms of mass COD/day).

- Methane production by a strain of Archaea growing on acetate follows this molar stoichiometry when cell yield is considered:

1CH3COO- + 0.03CO2 + 0.02NH4+ + 0.92H2O  0.95CH4 + 0.02C5H7NO2 + 0.98HCO3-

  - Write the mass-basis stoichiometry for methane production and show that mass conservation is satisfied.

  - Write the COD-basis stoichiometry for methane production and show that COD is conserved.

  - If acetate is consumed in an anaerobic digester at a rate of 500 kg-CH3COO-/d, at a temperature of 35 oC and a pressure of 1 atm, what is the rate of methane (CH4) production in m3/day, assuming the stoichiometry given above?

  - What is the rate of volatile suspended solids generation (kg-VSS/day) assuming 1 g C5H7NO2 = 1 g VSS (volatile suspended solids)?

- A new chemical plant is moving to your city and plans to discharge 1,000 m3/day of a wastewater containing 500 mg/l phenol (C6H5OH) as the primary component. Phenol is biodegradable under aerobic conditions. Prior to the new industrial influent the wastewater plant average inflow was 25,000 m3/day and the average influent COD is 450 mg/l.

  - What will the increase in COD loading be after the phenol wastewater is added?

  - If the cell yield on phenol is estimated to be 0.35 g-cell COD/b-phenol-COD, what is the additional oxygen requirement?

  - What else would you be concerned about before accepting this waste?

- For the following concurrent reactions and rate expressions,

  - develop a tableau formulation for the reactions and components by populating the cells in the table below. Leave cells blank when there is no appropriate coefficient

  - (next page)

Reaction 1: heterotrophic bacteria growth and aerobic mineralization of soluble COD

Reaction 1a: synthesis uptake of ammonia nitrogen by heterotrophic bacteria

-iNXBSNH + iNXBXNB = 0

Reaction 2: autotrophic bacteria growth on ammonia and energy production, neglect synthesis uptake of ammonia for autotrophs

Reaction 3: death, lysis and decay of heterotrophic bacteria and formation of debris and soluble substrate COD

(-1)XB + (1-fD)SS + fDXD = 0

Where components and stoichiometric coefficients are

XBH = heterotrophic bacteria cells (g-COD/m3)

XBA = autotrophic bacteria cells (g-COD/m3)

XD = nonbiodegradable cell debris (g-COD/m3)

XNB = nitrogen stored in cells (g-N/m3)

SS = substrate soluble COD (g-COD/m3)

SNH = soluble ammonia nitrogen (g-N/ m3)

SO = oxygen (g-O2/ m3)

iNXB = 0.08 g-N/g-cell-COD and iNXD = g-N/g-debris-COD

YH = 0.55 g-heterotroph cell-COD/g-COD consumed

YA = 0.12 g-autotroph cell-COD/g-NH4-N consumed

fD = 0.02 g debris-COD produced/g biomass-COD decayed

- Give the overall rate expressions for ammonia nitrogen, heterotrophic bacteria, autotrophic bacteria, debris, soluble COD, and oxygen considering the reactions 1, 1a, 2, and 3

COD and Nitrogen Stoichiometric and Kinetic Matrix FOR GROWTH AND DECAY

|  |  |  |  |  | Components |  |  |  |  |  |  | Rates |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Process | Soluble COD SS (mg/L COD) | Soluble NH4-N SNH (mg/L N) | Dissolved O2, SO (mg/L O2) | Heterotrophic biomass, XBH (mg/L COD) |  | Autotrophic (Nitrifying) Biomass, XBA (mg/L COD) |  |  | Debris, XD (mg/L COD) | Ammonia N in cells XNB (mg/L N) |  |  | rj |
| Aerobic Heterotrophic Growth |  |  |  |  |  |  |  |  |  |  |  |  | 1 and 1a HXBH |
| Aerobic Growth of Autotrophs (Nitrification) |  |  |  |  |  |  |  |  |  |  |  |  |  AXBA |
| Decay and Lysis of Heterotrophs |  |  |  |  |  |  |  |  |  |  |  |  | 3 bHXBH |

And reaction rates are

H*XBH = growth rate for heterotrophs (d-1)

A*XBA = growth rate for autotrophs (d-1)

bH*XBH = decay rate for heterotrophs (d-1)