CVEN 5534: Wastewater Treatment

Fall 2010

Assignment 4: Due Thursday, Oct. 14

- A wastewater containing 250 mg/l soluble COD is to be treated in a CSTR so that the effluent COD is less than 25 mg/l.  The flow rate is 10,000 m3/day

Q = 10,000 m3/d

So = 250 mg/l CODS

Xo = 0

Q = 10,000 m3/d

S = 25 mg/l CODS

X

V

S

X

Assuming Monod kinetics, steady-state operation of the CSTR and neglecting cell decay:

μ=μSKS+S and -rS=μXY

Where μ = max. specific cell growth rate = 2 d-1

S = soluble CODS concentration, mg/l

KS = half-saturation constant = 50 mg/l CODS

X = heterotrophic cell concentration, mg-CODX/l

Y = COD yield = 0.6 g-CODX/g-CODS

- What is the minimum hydraulic residence time,, before washout occurs?

- What reactor volume is required for 90% COD removal?

- What is the effluent cell concentration?

- Compare the volume of an ideal plug flow reactor required for the same level of COD removal and the same kinetic model.

- Plot S vs  for the CSTR and PFR

- Wastewater containing 300 g/m3 COD is to be treated for 85% COD removal.  The reaction rate is assumed to be first order with respect to COD concentration because the concentration of cells is constant.

rS = -kS

where: rS = rate of COD consumption (g/m3/d), k = first-order reaction coefficient = 5 d-1, and SS = COD concentration (g/m3)

- Compare the required system volumes for

  - An ideal single tank CSTR

  - An ideal plug flow reactor

  - A CSTR cascade with 2 cells

  - A CSTR cascade with 4 cells

- How will the ratio of required volumes change for 99% removal?

- Consider a CSTR where cells are perfectly separated from the effluent and are wasted in a separate smaller flow (Qw) which allows them to accumulate in the CSTR to a higher concentration than a CSTR with no enhanced cell retention.

Q = 10,000 m3/d

So = 250 mg/l CODS

Xo = 0

Q-Qw

S = 25 mg/l CODS

Xe = 0

V

S

X

Qw, S, X

Assuming Monod kinetics, now with decay,

rX = ( – b)X

and as in #1

μ=μSKS+S and -rS=μXY

Assume the same wastewater characteristics, effluent COD limit and kinetic parameters as in problem 1 and b = 0.1 d-1.

Also 2 parameters are now available to describe CSTR conditions:

τ=VQ and θ=VQw

Where  is called the average solids residence time (SRT).

- Write steady state mass balances on cells (X) and CODS (S) and solve to get formulas for

  - S as a function of the kinetic and CSTR parameters

  - X as a function of kinetic and CSTR parameters

- If the cell wasting flow rate, Qw = 300 m3/day what is the CSTR cell concentration and the required CSTR volume?

- Compare V and X to the values you got for the CSTR in problem 1.